Q1. Double Precision Multiplication

Suppose there are two double-precision floating point numbers X and Y. They are stored in memory locations at index 0 and 8, respectively, with the base address at X10.

Compute Z = X * Y and store it in the memory location at index 16

fld f1, 0(x10)       # load X (double)

fld f2, 8(x10)       # load Y (double)

fmul.d f3, f1, f2    # Z = X * Y

fsd f3, 16(x10)      # store Z (double)

Q2. Single Precision Division

Suppose there are two single-precision floating point numbers X and Y. They are stored in memory locations at index 0 and 4, respectively, with the base address at X11.

Compute Z = X / Y and store it in the memory location at index 8.

flw f1, 0(x11)       # load X (single)

flw f2, 4(x11)       # load Y (single)

fdiv.s f3, f1, f2    # Z = X / Y

fsw f3, 8(x11)      # store Z (single)

Q3. Double Precision Fused Multiply-Add

Assume you have three double-precision floating-point numbers A, B, and C stored in registers f1, f2, and f3. Compute A * B + C and store the result at memory location 24(X10).

fmadd.d f4, f1, f2, f3  	# f4 = (f1 * f2) + f3

fsd f4, 24(x10)     		# store f4

Q4. Integer to Single Precision Conversion

Given a 32-bit integer N, which is stored at index 12 of the base address of register X20. Convert it to a floating-point number.

lw x5, 12(x20)         # load the integer

fcvt.s.w f1, x5        # Convert integer to float

Q5. Move float to Integer Register

Assume a double-precision float is stored in f5. Move its bits into an integer register x28, add 10 to it, and store the result back in memory at 8(x10).

fmv.x.d x28, f5          # move float bits -> x28

addi x28, x28, 10         # integer addition

sd x28, 8(x10)           # store result